Brick Breaker Game - Complete Documentation

Course: Computer Organization and Assembly Language (COAL)

Project Title: Atari Breakout Implementation in x86 Assembly

Developed By:

- Imtinan ul Haq

- Ahmad Jameel

Date: December 2024

Table of Contents

- Project Overview

- Game Mechanics and Scoring

- System Architecture

- BIOS Interrupt Usage

- Program Flow and Logic

- Data Structures

- Key Functions

- Collision Detection System

- Technical Challenges and Solutions

- Flowcharts

1. Project Overview

This project presents a fully functional recreation of the classic Atari Breakout arcade game, implemented entirely in x86 assembly language for DOS environment. The game runs in real-mode 16-bit architecture and demonstrates low-level programming concepts including direct video memory manipulation, interrupt handling, and real-time input processing.

1.1 Technical Specifications

- Platform: x86 DOS (Real Mode)

- Assembly Syntax: NASM

- Video Mode: Text Mode 80x25

- Memory Model: COM executable (ORG 100h)

- Input Method: BIOS keyboard interrupts

- Display Method: BIOS video services

1.2 Game Features

The implementation includes several sophisticated features that demonstrate advanced assembly programming techniques:

- Dynamic paddle control with smooth horizontal movement

- Physics-based ball trajectory with variable angles

- Multi-colored brick arrangement with differential scoring

- Real-time collision detection across multiple object types

- Sound feedback through PC speaker manipulation

- Live score tracking and life counter display

- Win/loss condition detection and appropriate game state transitions

2. Game Mechanics and Scoring

2.1 Gameplay Rules

The player controls a horizontal paddle positioned near the bottom of the screen. A ball continuously moves around the playing field, bouncing off walls and bricks. The objective is to destroy all bricks by making the ball collide with them while preventing the ball from falling past the paddle.

Core Mechanics:

- Players use left and right arrow keys to move the paddle horizontally

- The ball's horizontal trajectory changes based on where it hits the paddle

- Hitting the left third of the paddle sends the ball leftward at an angle

- Hitting the center keeps the ball moving vertically

- Hitting the right third sends the ball rightward at an angle

- Each brick requires one hit to destroy

- Missing the ball costs one life

- The game ends when all lives are lost or all bricks are destroyed

2.2 Scoring System

The game implements a row-based scoring mechanism that rewards players differently based on brick position. Bricks closer to the paddle (bottom rows) offer fewer points, while those farther away provide higher rewards.

Point Values:

| Row Position | Color | Points per Brick |
| --- | --- | --- |
| Row 1 (Top) | Blue | 40 points |
| Row 2 | Cyan | 30 points |
| Row 3 | Yellow | 20 points |
| Row 4 (Bottom) | Red | 10 points |

This scoring distribution encourages strategic gameplay, as players must decide whether to aim for easier low-value targets or risk more difficult shots for higher scores.

2.3 Brick Layout

The playing field contains 64 total bricks arranged in a specific pattern:

- 4 horizontal rows of bricks

- 16 bricks per row

- Each brick occupies 4 character cells

- Bricks are separated by 1 character gap

- Pattern repeats: [BBBB]_[BBBB]_[BBBB]_ (B = brick cell, _ = gap)

This layout spans nearly the entire width of the 80-column screen, creating a challenging but achievable target area.

2.4 Lives and Game Progression

Players begin with 3 lives. Each time the ball falls past the paddle:

- One life is subtracted

- Ball position resets to center screen

- Ball velocity resets to upward vertical movement

- Paddle remains in its current position

The game continues until either:

- All lives are depleted (defeat condition)

- All bricks are destroyed (victory condition)

3. System Architecture

3.1 Memory Organization

The program uses the COM executable format, which loads at memory offset 100h. This simple memory model places code and data in the same segment, allowing for straightforward addressing.

Memory Layout:

0000h - 00FFh: DOS Program Segment Prefix (PSP)

0100h - CODE_START: Program entry point

DATA SECTION: Game state variables and constants

TEXT SECTION: Executable code

STACK: Grows downward from top of segment

3.2 Data Segment Structure

All game state is maintained in memory-resident variables that persist throughout execution. Key data categories include:

Display Constants:

- Character codes for rendering game objects

- Color attribute bytes for visual distinction

- Symbol definitions for paddle, ball, and bricks

Game State Variables:

- Paddle position (column, row) and dimensions

- Ball position (column, row) and velocity vectors

- Player statistics (score, lives remaining)

- Brick destruction tracking array

Configuration Parameters:

- Screen boundary definitions

- Movement speed constants

- Point value tables

3.3 Code Organization

The codebase follows a modular structure with clearly defined functional sections:

- Initialization Module - Sets video mode, initializes variables

- Main Game Loop - Continuous execution cycle handling input and updates

- Physics Engine - Ball movement and boundary collision

- Collision Detection - Paddle and brick intersection testing

- Rendering System - Screen drawing and updates

- Audio Subsystem - PC speaker sound generation

- UI Management - Score display and menu screens

4. BIOS Interrupt Usage

The program relies heavily on BIOS interrupts to interface with hardware. These interrupts provide standardized methods for keyboard input, video output, timing, and sound generation.

4.1 Video Services (INT 10h)

INT 10h, AH=00h - Set Video Mode

- Used at startup to initialize 80x25 text mode

- AL=03h selects standard color text mode

- Clears screen and resets cursor position

INT 10h, AH=01h - Set Cursor Shape

- Makes cursor invisible during gameplay

- CH=20h sets cursor to invisible mode

- Prevents cursor from interfering with game graphics

INT 10h, AH=02h - Set Cursor Position

- Positions cursor before character output

- DH=row, DL=column (0-indexed)

- Essential for placing text at specific screen coordinates

- Called before every character write operation

INT 10h, AH=06h - Scroll Window Up

- Used with AL=0 to clear entire screen

- BH=07h sets normal white-on-black attributes

- CH/CL define top-left corner (0,0)

- DH/DL define bottom-right corner (24,79)

- Faster than individually clearing each character

INT 10h, AH=09h - Write Character with Attribute

- Writes single character at current cursor position

- AL=character code to display

- BL=color attribute byte

- CX=repetition count (usually 1)

- Does not advance cursor automatically

INT 10h, AH=0Eh - Teletype Output

- Used for number display in score output

- AL=character to print

- Automatically advances cursor

- Simpler than attribute-based output for sequential text

4.2 Keyboard Services (INT 16h)

INT 16h, AH=00h - Wait for Keystroke

- Blocks execution until key is pressed

- Returns scan code in AH, ASCII code in AL

- Used in menus and end-game screens

- Ensures user input is captured

INT 16h, AH=01h - Check Keyboard Status

- Non-blocking keyboard check

- Zero flag set if no key waiting

- Zero flag clear if key available in buffer

- Allows game loop to continue without waiting

- Critical for maintaining smooth gameplay

4.3 Timer Services (INT 1Ah)

INT 1Ah, AH=00h - Read System Time

- Returns BIOS timer tick count

- CX:DX contains 32-bit tick value

- Ticks increment approximately 18.2 times per second

- Used to create consistent frame delays

- Ensures game speed remains constant across different CPU speeds

Implementation in pause_frame:

mov ah, 00h

int 1Ah         ; Get current time in DX

mov bx, dx

add bx, 2       ; Add delay (2 ticks ≈ 110ms)

frame_wait:

int 1Ah     ; Keep checking time

cmp dx, bx

jl frame_wait   ; Loop until target time reached

4.4 DOS Services (INT 21h)

INT 21h, AH=09h - Display String

- Prints dollar-terminated string

- DS:DX points to string

- Stops at '$' character

- Used for menus, messages, and labels

- Simple but effective for text output

INT 21h, AH=4Ch - Terminate Program

- Cleanly exits to DOS

- AL=exit code (00h = success)

- Closes files and releases memory

- Proper way to end COM programs

4.5 Direct Hardware Access

PC Speaker Control (Ports 42h, 43h, 61h)

The program directly manipulates hardware ports to generate sound effects:

Port 43h (PIT Mode Control):

mov al, 182     ; 10110110b

; Channel 2, LSB/MSB, Square wave, Binary mode

out 43h, al

Port 42h (Timer Channel 2):

mov ax, frequency_divisor

out 42h, al     ; Send LSB

mov al, ah

out 42h, al     ; Send MSB

Port 61h (PPI Port B - Speaker Control):

in al, 61h      ; Read current value

or al, 03h      ; Set bits 0-1 (enable speaker + timer gate)

out 61h, al     ; Activate sound

; ... delay ...

in al, 61h      ; Read current value

and al, 0FCh    ; Clear bits 0-1 (disable speaker)

out 61h, al     ; Silence

Sound Frequencies Used:

- Paddle bounce: 1000 Hz (1193180 / 1000 = 1193 divisor)

- Brick destruction: 2000 Hz (596 divisor)

- Life lost: 500 Hz (2386 divisor)

5. Program Flow and Logic

5.1 High-Level Program Structure

The program follows a traditional game loop architecture with distinct phases:

START

↓

Initialize System (Video, Cursor)

↓

Display Welcome Screen

↓

Wait for User Input (Enter/ESC)

↓

Initialize Game State

↓

╔═══════════════════════════════════╗

║      MAIN GAME LOOP               ║

║  1. Check keyboard input          ║

║  2. Update ball position          ║

║  3. Check collisions              ║

║  4. Update display                ║

║  5. Check win/loss conditions     ║

║  6. Delay for frame timing        ║

╚═══════════════════════════════════╝

↓

Display End Screen (Win/Loss)

↓

Wait for ESC

↓

Restore Video Mode

↓

EXIT

5.2 Game State Initialization

Before entering the main loop, the game performs comprehensive state setup:

Variable Initialization:

- player_score = 0 - Reset score counter

- remaining_lives = 3 - Set starting lives

- bar_col = 35 - Center paddle horizontally

- sphere_col = 40, sphere_row = 20 - Position ball above paddle

- sphere_h_speed = 0, sphere_v_speed = -1 - Set initial upward velocity

- blocks_left = 0 - Reset brick counter

Brick State Reset: A 64-byte array tracks whether each brick exists:

mov cx, 64              ; 64 bricks total

mov di, brick_states    ; Point to array

reset_bricks:

mov byte [di], 1    ; Mark as active

inc di

loop reset_bricks

Initial Rendering:

- Draw all brick rows with appropriate colors

- Render paddle at starting position

- Display initial score and lives

- Position ball on screen

5.3 Main Game Loop Detailed Flow

The core game loop executes continuously until a win or loss condition occurs:

Phase 1: Input Processing

Check if key pressed (INT 16h, AH=01h)

↓

If no key → Skip to Phase 2

↓

If key available:

- Read key (INT 16h, AH=00h)

- If ESC → Exit game

- If Left Arrow (scan code 4Bh):

* Erase paddle at current position

* Decrease bar_col by 2

* Clamp to minimum 0

- If Right Arrow (scan code 4Dh):

* Erase paddle at current position

* Increase bar_col by 2

* Clamp to maximum (80 - paddle_width)

Phase 2: Physics Update

Erase ball from old position

↓

Update ball position:

sphere_col += sphere_h_speed

sphere_row += sphere_v_speed

↓

Check wall collisions:

- Left wall (col = 0) → Reflect horizontally

- Right wall (col = 79) → Reflect horizontally

- Top wall (row = 0) → Reflect vertically

- Bottom wall (row = 24) → Lose life, reset ball

Phase 3: Collision Detection

Check paddle collision:

If ball_row == paddle_row AND

ball_col >= paddle_left AND

ball_col <= paddle_right:

- Calculate hit position (left/center/right third)

- Set horizontal speed based on hit location

- Set vertical speed to -1 (upward)

- Play bounce sound

↓

Check brick collision:

If ball in brick area (rows 2-5):

- Calculate brick column: ball_col / 5

- Skip if in gap (ball_col % 5 == 4)

- Calculate brick ID: (row-2) * 16 + column

- If brick active:

* Mark brick as destroyed

* Award points based on row

* Erase brick from screen

* Decrease blocks_left counter

* Reflect ball vertically

* Play brick sound

Phase 4: Rendering

Render paddle at current position (every frame)

Render ball at new position

Update score and lives display

Phase 5: Game State Check

If remaining_lives == 0:

→ Jump to defeat screen

↓

If blocks_left == 0:

→ Jump to victory screen

↓

Otherwise:

→ Continue loop

Phase 6: Frame Timing

Call pause_frame:

- Read system timer

- Add delay (2 ticks ≈ 110ms)

- Wait until timer reaches target

→ Ensures consistent 9 FPS regardless of CPU speed

5.4 Collision Detection Logic

Paddle Collision Algorithm:

1. Check if ball row matches paddle row

2. Check if ball column is within paddle bounds

3. If collision detected:

a. Calculate paddle_width / 3 to define sections

b. Determine which third was hit:

- Left third (0 to width/3): h_speed = -2

- Center third (width/3 to 2*width/3): h_speed = 0

- Right third (2*width/3 to width): h_speed = +2

c. Always set v_speed = -1 (upward)

d. Generate bounce sound

Brick Collision Algorithm:

1. Verify ball is in brick zone (rows 2-5)

2. Calculate brick column:

column = ball_col / 5

offset = ball_col % 5

3. If offset == 4, ball is in gap → no collision

4. Calculate brick index:

row_offset = (ball_row - 2) * 16

brick_id = row_offset + column

5. Validate brick_id < 64 (safety check)

6. Check brick_states[brick_id]:

- If 0: already destroyed → skip

- If 1: active brick → process collision

7. Process collision:

a. Set brick_states[brick_id] = 0

b. Decrement blocks_left

c. Lookup points in row_points table

d. Add points to player_score

e. Erase 4 cells from screen

f. Reverse v_speed (bounce)

g. Generate brick sound

6. Data Structures

6.1 Brick State Array

Purpose: Track which bricks have been destroyed

Structure:

brick_states    times 64 db 1

Layout:

- 64 bytes total (one per brick)

- Each byte represents one brick's state:

  - Value 1 = brick exists

  - Value 0 = brick destroyed

- Organized as 4 rows × 16 columns

Indexing Formula:

brick_id = (row - 2) * 16 + column

where:

row = ball's screen row (2-5)

column = ball_col / 5 (0-15)

Example:

- Brick at row 3, column 5:

  - ID = (3-2) * 16 + 5 = 21

  - Access: brick_states[21]

6.2 Point Value Table

Purpose: Define scoring for each brick row

Structure:

row_points dw 40, 30, 20, 10

Usage:

mov al, byte [sphere_row]  ; Get ball row

sub al, 2                   ; Convert to 0-3

xor ah, ah

mov si, ax

shl si, 1                   ; Multiply by 2 (word size)

mov ax, [row_points + si]   ; Load points

add [player_score], ax      ; Award score

6.3 Game State Variables

Paddle Configuration:

bar_col     dw 35    ; Horizontal position (0-69)

bar_row     dw 23    ; Vertical position (fixed)

bar_size    dw 10    ; Width in characters

bar_color   db 0x3F  ; Cyan on white background

Ball State:

sphere_col      dw 40     ; X position (0-79)

sphere_row      dw 20     ; Y position (0-24)

sphere_h_speed  dw 0      ; Horizontal velocity (-2, 0, +2)

sphere_v_speed  dw -1     ; Vertical velocity (-1, +1)

sphere_color    db 0x0F   ; Bright white

Player Statistics:

player_score     dw 0      ; Current score (0-65535)

remaining_lives  dw 3      ; Lives left (0-3)

blocks_left      dw 0      ; Active bricks remaining

7. Key Functions

7.1 Movement Functions

shift_left - Move Paddle Left

Purpose: Moves paddle 2 columns left with boundary checking

Input: None (reads bar_col)

Output: Updates bar_col

Algorithm:

1. Check if bar_col <= 0

2. If yes, ignore input (at left edge)

3. Erase paddle at current position

4. Subtract 2 from bar_col

5. Clamp to minimum 0 if negative

shift_right - Move Paddle Right

Purpose: Moves paddle 2 columns right with boundary checking

Input: None (reads bar_col, bar_size)

Output: Updates bar_col

Algorithm:

1. Calculate right edge: bar_col + bar_size

2. Check if >= 80 (screen width)

3. If yes, ignore input (at right edge)

4. Erase paddle at current position

5. Add 2 to bar_col

6. Clamp to maximum (80 - bar_size) if exceeds

update_sphere - Ball Movement and Collision

Purpose: Updates ball position and handles all collisions

Input: None (reads ball state variables)

Output: Updates sphere_col, sphere_row, velocities, game state

Algorithm:

1. Add sphere_h_speed to sphere_col

2. Add sphere_v_speed to sphere_row

3. Check left wall (col = 0)

- Clamp position, negate h_speed, play sound

4. Check right wall (col = 79)

- Clamp position, negate h_speed, play sound

5. Check ceiling (row = 0)

- Clamp position, negate v_speed, play sound

6. Check floor (row = 24)

- Decrement lives, reset ball, play loss sound

7. Check paddle collision

- Calculate hit section, adjust trajectory

8. Check brick collision

- Identify brick, destroy if exists, award points

7.2 Rendering Functions

render_blocks - Draw All Bricks

Purpose: Renders 4 rows of bricks across screen width

Input: None

Output: Draws bricks to screen, updates blocks_left

Algorithm:

For each row (2-5):

1. Set color based on row:

- Row 2: Blue (0x09)

- Row 3: Cyan (0x0B)

- Row 4: Yellow (0x0E)

- Row 5: Red (0x0C)

2. Start at column 0

3. While space remains:

a. Draw 4-cell brick

b. Increment blocks_left

c. Skip 1 gap cell

d. Repeat until row full

render_bar - Draw Paddle

Purpose: Draws paddle at current position

Input: bar_col, bar_row, bar_size, bar_color

Output: Paddle displayed on screen

Algorithm:

1. Position cursor at (bar_row, bar_col)

2. Set color to bar_color

3. Loop bar_size times:

a. Write bar_symbol character

b. Move cursor right

erase_bar - Clear Paddle

Purpose: Removes paddle from screen

Input: bar_col, bar_row, bar_size

Output: Paddle area cleared

Algorithm:

1. Position cursor at (bar_row, bar_col)

2. Loop bar_size times:

a. Write space character (black color)

b. Move cursor right

Note: Called before moving paddle to prevent trails

render_sphere - Draw Ball

Purpose: Displays ball at current position

Input: sphere_col, sphere_row, sphere_color

Output: Ball displayed on screen

Algorithm:

1. Position cursor at (sphere_row, sphere_col)

2. Write sphere_symbol ('O') with sphere_color

show_stats - Display Score and Lives

Purpose: Updates status line with current game statistics

Input: player_score, remaining_lives

Output: Top screen line shows "Points: XXX  Lives: X"

Algorithm:

1. Clear entire row 0 with spaces (prevent overlap)

2. Position cursor at (0, 0)

3. Print "Points: " label

4. Call display_number with player_score

5. Position cursor at (0, 60)

6. Print " Lives: " label

7. Call display_number with remaining_lives

7.3 Utility Functions

position_cursor - Set Cursor Location

Purpose: Moves cursor to specific screen position

Input: DH=row (0-24), DL=column (0-79)

Output: Cursor positioned

Interrupt: INT 10h, AH=02h, BH=00h

place_character - Write Single Character

Purpose: Writes one character with color at current position

Input: AL=character, BL=color, DH=row, DL=column

Output: Character displayed

Algorithm:

1. Call position_cursor with DH, DL

2. Write character with INT 10h, AH=09h

3. Restore registers (non-destructive)

erase_display - Clear Entire Screen

Purpose: Fills screen with spaces

Input: None

Output: Blank screen with black background

Interrupt: INT 10h, AH=06h, AL=0

CH=0, CL=0 (top-left)

DH=24, DL=79 (bottom-right)

BH=07h (white on black)

pause_frame - Timing Delay

Purpose: Creates consistent frame delay

Input: None

Output: Approximately 110ms delay

Algorithm:

1. Read timer (INT 1Ah, AH=00h) → DX

2. Add 2 ticks to DX → BX

3. Loop:

a. Read timer again

b. Compare to target

c. Repeat if not reached

Effect: Maintains ~9 FPS game speed

display_number - Convert Integer to ASCII

Purpose: Displays decimal number at cursor position

Input: AX=number to display (0-65535)

Output: Number printed as ASCII text

Algorithm:

1. Handle zero case → print '0'

2. Loop:

a. Divide AX by 10

b. Push remainder (digit)

c. Increment digit counter

d. Repeat until AX=0

3. Pop and print each digit:

a. Add '0' to convert to ASCII

b. Output with INT 10h, AH=0Eh

7.4 Audio Functions

audio_bounce - Paddle/Wall Bounce Sound

Purpose: Generates 1000 Hz tone

Input: None

Output: Brief beep from PC speaker

Frequency: 1000 Hz (pleasant mid-tone)

audio_brick - Brick Destruction Sound

Purpose: Generates 2000 Hz tone

Input: None

Output: Higher-pitched beep

Frequency: 2000 Hz (satisfying high tone)

audio_loss - Life Lost Sound

Purpose: Generates 500 Hz tone

Input: None

Output: Low-pitched warning beep

Frequency: 500 Hz (ominous low tone)

create_sound - PC Speaker Control

Purpose: Generates tone of specified frequency

Input: BX=frequency divisor

Output: Brief tone through speaker

Algorithm:

1. Configure PIT channel 2:

- OUT 43h, 182 (square wave mode)

2. Set frequency:

- OUT 42h, BL (LSB of divisor)

- OUT 42h, BH (MSB of divisor)

3. Enable speaker:

- IN from port 61h

- OR with 03h

- OUT to port 61h

4. Delay (CX=0200h loop)

5. Disable speaker:

- IN from port 61h

- AND with 0FCh

- OUT to port 61h

8. Collision Detection System

8.1 Paddle Collision

Detection Criteria:

- Ball must be at same row as paddle (sphere_row == bar_row)

- Ball column must be within paddle bounds:

  - sphere_col >= bar_col

  - sphere_col < bar_col + bar_size

Trajectory Calculation:

The paddle is divided into three equal sections, each producing different ball angles:

Paddle Layout (10 cells wide):

┌────┬────┬────┐

│ L  │ C  │ R  │  L=Left third (cells 0-2)

│ E  │ E  │ I  │  C=Center third (cells 3-6)

│ F  │ N  │ G  │  R=Right third (cells 7-9)

│ T  │ T  │ H  │

│    │ E  │ T  │

│    │ R  │    │

└────┴────┴────┘

-2    0    +2   ← Horizontal speed values

Implementation:

; Calculate relative hit position

mov ax, [sphere_col]

sub ax, [bar_col]       ; AX = offset within paddle (0 to bar_size-1)

; Divide paddle width by 3

mov bx, [bar_size]

mov dx, 0

mov cx, 3

div cx                  ; AX = section number (0, 1, or 2)

; Apply trajectory based on section

cmp ax, 0

je hit_left_third       ; Section 0 → go left

cmp ax, 1

je hit_center_third     ; Section 1 → go straight

; Section 2 → go right

hit_left_third:

mov word [sphere_h_speed], -2

jmp bounce_up

hit_center_third:

mov word [sphere_h_speed], 0

jmp bounce_up

; Right third (fall through)

mov word [sphere_h_speed], 2

bounce_up:

mov word [sphere_v_speed], -1

call audio_bounce

8.2 Brick Collision

Step 1: Verify Ball in Brick Zone

mov al, byte [sphere_row]

cmp al, 2               ; Row 2 is topmost brick row

jl no_brick_hit         ; Above bricks

cmp al, 5               ; Row 5 is bottom brick row

jg no_brick_hit         ; Below bricks

Step 2: Calculate Brick Column

mov al, byte [sphere_col]   ; AL = 0-79

xor ah, ah

mov bl, 5                   ; Each brick spans 5 chars (4 cells + 1 gap)

div bl                      ; AL = brick column (0-15)

; AH = offset within pattern (0-4)

cmp ah, 4                   ; Check if in gap

je no_brick_hit             ; Offset 4 means gap cell

Step 3: Calculate Brick Array Index

; AL currently contains brick column (0-15)

mov bl, al                  ; Save column

mov al, byte [sphere_row]

sub al, 2                   ; Convert row 2-5 to 0-3

mov cl, 4

shl al, cl                  ; Multiply by 16 (left shift 4 bits)

add al, bl                  ; Add column offset

; AL now contains brick ID (0-63)

Step 4: Check Brick State

xor bh, bh

mov bl, al                  ; BX = brick ID

cmp byte [brick_states + bx], 0

je no_brick_hit             ; Already destroyed

Step 5: Process Destruction

; Mark as destroyed

mov byte [brick_states + bx], 0

dec word [blocks_left]

; Award points

mov al, byte [sphere_row]

sub al, 2                   ; Row index (0-3)

xor ah, ah

mov si, ax

shl si, 1                   ; Word offset

mov ax, [row_points + si]   ; Load points

add word [player_score], ax

; Erase brick visually (4 cells)

[... erasing code ...]

; Bounce ball

neg word [sphere_v_speed]

8.3 Wall Collision

check_right_wall ; Not at left edge mov word [sphere_col], 0 ; Clamp to boundary neg word [sphere_h_speed] ; Reverse horizontal direction call audio_bounce

**Right Wall:**

```assembly

cmp word [sphere_col], 79

jl check_ceiling            ; Not at right edge

mov word [sphere_col], 79   ; Clamp to boundary

neg word [sphere_h_speed]   ; Reverse horizontal direction

call audio_bounce

Ceiling:

cmp word [sphere_row], 0

jg check_floor              ; Not at ceiling

mov word [sphere_row], 0    ; Clamp to boundary

neg word [sphere_v_speed]   ; Reverse vertical direction

call audio_bounce

Floor (Life Loss):

mov ax, [max_rows]

dec ax                      ; AX = 24

cmp [sphere_row], ax

jl check_bar_collision      ; Not at floor

dec word [remaining_lives]

call audio_loss

; Reset ball to center

mov word [sphere_col], 40

mov word [sphere_row], 20

mov word [sphere_v_speed], -1

mov word [sphere_h_speed], 0

9. Technical Challenges and Solutions

9.1 Challenge: Paddle Disappearing During Movement

Problem: When the paddle moved, some cells would not render, causing parts of the paddle to become invisible.

Root Cause: The render_bar function was only called after certain events, not every frame. This meant that if ball rendering or other operations overwrote paddle cells, they wouldn't be restored until the next explicit render call.

Solution: Modified the main loop to call render_bar every single frame, after ball updates but before the frame delay. This ensures the paddle is always fully visible regardless of what other rendering operations occur.

main_cycle:

; ... input processing ...

; ... ball update ...

call render_bar         ; ← Added: render EVERY frame

call render_sphere

call show_stats

; ... game state checks ...

jmp main_cycle

9.2 Challenge: Brick Collision Detection Gaps

Problem: Ball would sometimes pass through bricks without registering collision.

Root Cause: The gap between bricks (column position % 5 == 4) was not being properly excluded from collision detection.

Solution: Added explicit gap detection:

mov al, byte [sphere_col]

xor ah, ah

mov bl, 5

div bl          ; AH = remainder (0-4)

cmp ah, 4       ; Is ball in gap cell?

je no_brick_hit ; Yes → no collision possible

9.3 Challenge: Score Display Overlap

Problem: When score increased from single digit (9) to double digit (10), the display would show garbled text like "Points: 109" instead of "Points: 10".

Root Cause: New text was written over old text without clearing, causing overlap.

Solution: Modified show_stats to clear the entire status line before writing new values:

; Clear entire row 0 first

mov dh, 0

mov dl, 0

call position_cursor

mov cx, 80              ; Full screen width

mov al, ' '

mov bl, 0x07

clear_status_line:

push cx

call place_character

inc dl

pop cx

loop clear_status_line

; Now write fresh statistics

9.4 Challenge: Inconsistent Game Speed

Problem: Game ran at wildly different speeds on different computers.

Root Cause: No frame delay meant the game loop executed as fast as the CPU allowed.

Solution: Implemented timer-based frame delay using BIOS timer ticks:

pause_frame:

mov ah, 00h

int 1Ah             ; Get current tick count

mov bx, dx

add bx, 2           ; Target = current + 2 ticks (~110ms)

frame_wait:

int 1Ah             ; Check timer again

cmp dx, bx

jl frame_wait       ; Loop until target reached

ret

This creates a consistent ~9 FPS across all systems.

9.5 Challenge: Array Bounds Safety

Problem: Potential for array overflow if calculated brick index exceeded array size.

Root Cause: No validation of computed brick index before accessing brick_states array.

Solution: Added bounds checking:

; After calculating brick ID in AL

cmp al, 64          ; Maximum valid index

jge no_brick_hit    ; Out of bounds → ignore

; Safe to access array now

xor bh, bh

mov bl, al

cmp byte [brick_states + bx], 0

10. Flowcharts

10.1 Main Program Flow

10.2 Ball Movement and Collision System

10.3 Paddle Movement Logic

10.4 Brick Collision Detection Detail

10.5 Game Initialization Flow

10.6 Score Display and Statistics Update

10.7 Sound Generation System---
(All are in the Flow Chart Folder)

Appendix A: Color Attribute Reference

The game uses BIOS color attributes (format: background << 4 | foreground):

| Color Code | Background | Foreground | Usage |
| --- | --- | --- | --- |
| 0x09 | Black | Bright Blue | Top brick row |
| 0x0B | Black | Bright Cyan | Second brick row |
| 0x0E | Black | Bright Yellow | Third brick row |
| 0x0C | Black | Bright Red | Bottom brick row |
| 0x3F | Cyan | White | Paddle |
| 0x0F | Black | Bright White | Ball |
| 0x07 | Black | Light Gray | Text |

Appendix B: Memory Map

Segment Layout (COM format):

0000:0000 - Program Segment Prefix (256 bytes)

0000:0100 - Code Start (_start)

0000:XXXX - Data Section (variables, messages)

0000:YYYY - Stack (grows downward)

Variable Memory Addresses (relative to data section):

- player_score: 2 bytes

- remaining_lives: 2 bytes

- brick_states: 64 bytes

- sphere_col, sphere_row: 4 bytes total

- bar_col, bar_row, bar_size: 6 bytes total

Appendix C: Interrupt Summary Table

| Interrupt | Function | Purpose in Game |
| --- | --- | --- |
| INT 10h/00h | Set Video Mode | Initialize 80x25 text mode |
| INT 10h/01h | Set Cursor | Hide cursor |
| INT 10h/02h | Set Position | Position for character write |
| INT 10h/06h | Scroll Window | Clear screen |
| INT 10h/09h | Write Char+Attr | Draw game objects |
| INT 10h/0Eh | Teletype Output | Print numbers |
| INT 16h/00h | Read Keystroke | Menu input |
| INT 16h/01h | Check Keyboard | Non-blocking input |
| INT 1Ah/00h | Read Timer | Frame timing |
| INT 21h/09h | Display String | Print messages |
| INT 21h/4Ch | Terminate | Exit program |

Appendix D: Control Mappings

| Key | Scan Code | Action |
| --- | --- | --- |
| Left Arrow | 4Bh | Move paddle left 2 cells |
| Right Arrow | 4Dh | Move paddle right 2 cells |
| Enter | 0Dh | Start game (menus) |
| ESC | 1Bh | Exit game |

Conclusion

This Brick Breaker implementation demonstrates comprehensive understanding of x86 assembly programming, including interrupt handling, real-time input processing, collision detection algorithms, and direct hardware manipulation. The project successfully recreates the classic arcade experience while showcasing low-level programming techniques essential to computer organization and assembly language development.

The modular code structure, combined with robust error handling and consistent frame timing, results in a stable and enjoyable gaming experience. The detailed documentation provided here serves as both a technical reference and a learning resource for understanding how classic games were developed on early PC hardware.